KCDS Chart - Revision 2

Changes in current revision:

KCDS chart for use on Powertrain drawings has undergone some changes to facilitate commonality with VEC operations. The main changes are:

- Reference to standard in the block changed from GM-1805 to QN-1805

- Changes to Part Level Requirements portion of the chart

  - S - changed from SAFETY to SAFETY COMPLIANCE

  - C - changed from COMPLIANCE to CERTIFICATION

  - Acronym F/C changed to F

- "C/H - Component Handling" removed as a Part Level Requirement

- Unique numbering of KPCs and PQCs in the chart - KPCs and PQCs need to follow their own unique numbering sequence. The current practice of common numbering sequence for both KPCs and PQCs is no longer valid

There are no changes to KPC and PQC symbol representations on the drawing.

Usage in NX:

KCDS chart and symbols are placed on the drawing using "Standard Notes" program of GMPDL:

- With the drawing on display, run the program

Tools -> GMPDL 1-B -> Annotate -> Standard Notes

- Click on Choose Standard Note

- Click on Choose from STD directory

- Select one of - kcds_v2_block, kcds_v2_block_11x17, kcds_kpcnum or kcds_pqcnum - as appropriate and click on OK.

- Indicate the top right corner for the chart and center point for the symbols to place them on the drawing.

- Click on Keep It to confirm placement of the chart/symbol.

- Exit the program.

The above steps ensure that the chart and the symbols are placed on the drawing on the work layer. Select the text fields represented by dashes to fill appropriate information using the Annotation Editor of NX. It is important to use the existing text in the chart or the symbols while entering or editing the fields to maintain proper character height and aspect ratio.

Legacy files with Rev.1 of KCDS chart:

Powertrain design management has decided not to revise legacy drawings with Rev.1 of the KCDS chart. Rev.2 template will be used only if the drawing is revised to have a new part/item number. You are advised to follow this instruction or an appropriate direction from your supervisor/DLT.

How to handle the old KPC block:

The drawing formats created by the current version of GMPDL still have built-in KPC blocks of the old kind, which are not valid if you are using the KCDS system. The preferred method to override the KPC block with the KCDS chart is:

- Cross-out the old KPC block on the face of the drawing and place the KCDS chart underneath it

Other methods such as blanking or moving the KPC block to another layer are not acceptable, as these do not conform to Powertrain best practices and Data Creation Standards.

NEVER delete the old KPC block to make way for the KCDS charts, as this will likely result in the corruption of drawing formats.